HanseView 2.1.40 Installation Notes:

Before Installation please record all PID settings. These can be accessed via HanseView 2.x under the Configuration Menu

After Installation Please follow steps listed below:

1. If you use a Watlow F4 controller you must set the Cascade style in your Configuration -> Parameters. Directly next to controller selector. Our equipment all uses Deviation Cascade. DO NOT USE PROCESS CASCADE UNLESS SPECIFIED TO DO SO BY HANSE ENVIRONMENTAL INC.

2A. Make sure to set your Max Heat and Cool Boost in the PID settings. We suggest    10 and -10 to start with. What this number allows is the Air temperature to be above or below the set-point by the specified amounts.